ECE485 Computer Architecture

Lesson 29 Worksheet

- Recall that:

Total Latency = Overhead + Trans Time

Total Latency = Sender Overhead + Time of Flight + Transmission Time + Receiver Overhead

Given the following scenario, what is the total latency for the transmission?  What percentage of the transmission time is due to overhead?

  - Bandwidth = 10 Mbits/Sec

  - Sender Overhead = 230 microsec

  - Receiver Overhead = 270 microsec

  - Message Size = 1000 Bytes

  - Signal Propagation = 50% of Speed of Light

  - Distance = 100 Meters